Blue connector

1 Ground BLK/WHT
2 Ignition LT GREEN	(to crank position sensor)
3 Inj 3 PKN/BLK
4 Inj 2 LT GRN/BLK
5   
6 
7 
8 
9 
10 
11 KS Signal LT BLU
12 CKP DK BLU/WHT (to crank pos. sensor)
13 Req torque ORG/BLK (to electronic brake control module)
14 serial data ORG/BLK  (to throttle actuator control module, fly by wire throttle)
15 serial data DK BLU/WHT  (to throttle actuator control module, fly by wire throttle)
16 
17 range trans fluid pressure b DK BLU (to transmission)
18 range trans fluid pressure c RED  (to transmission)
19 ignition PNK  (to fuse center)
20 B+ ORG/BLK (to fuse center, constant 20A fuse, ECMB fuse)
21 Ref low YEL  (crank pos. sensor)
22 
23 sens gnd PPL (EGR valve)
24 
25 HO2S LO TAN (bank 2 sensor 1)
26 HO2S LO TAN (bank 2 sensor 1)
27 
28 HO2S LO TAN/WHT (bank 2 sensor 1)
29 HO2S LO TAN/WHT (bank 2 sensor 1)
30 Coolant level LT GRN (on radiator)
31 
32 PRND A BLK/WHT  (trans range sensor signal a)
33 BRAKE TCC SW PPL (to black 14 pin connector then brake switch in car)
34 PRND P WHT (trans range signal p)
35 
36 Inj 1 BLK/WHT
37 Inj 6 YEL/BLK
38 Lift Dive sig PNK/WHT (to suspension control module)
39 
40 Ground BLK/WHT
41 Ground BLK (to transmission fluid temp sensor)
42 
43 inj 7 RED/BLK
44 inj 4 LT BLU/BLK
45 +5V GRY (Fuel tank pressure sensor)
46 +5V GRY (TPS)
47 +5V GRY (MAP)
48 +5V GRY (EGR)
49 
50 
51 KS 1 Signal 
52 
53 sens gnd (?)
54 sens gnd (?)
55 EGR input 
56 Fuel Comp Sig (?)
57 B+ ORG (to fuse center, constant 20A fuse, ECMB fuse)
58 CLS 2 Data DK GRN (to OBD II diagnostic port pin 2)
59 CLS 2 Data YEL  (to BCM fuel enable signal, serial data, VATS must be deleted in truck computers.)
60 sens gnd (?)
61 Ref low  (?)
62 
63 sens gnd (?)
64 
65 HO2S HI  (bank 2 sensor 2)
66 HO2S HI (bank 2 sensor 1)
67 
68 HO2S HI  (bank 2 sensor 1)
69 HO2S HI  (bank 1 sensor 1)
70 Oil Level BRN (to oil switch in pan)
71 
72 PRND B (trans range signal B)
73 CMP BRN/RED or BRN/WHT (to cam pos sensor)
74 ECT sig  YEL  (coolant temp signal, to sender in head)
75 Ignition PNK (to fuse center)
76 inj 5 BLK/WHT
77 inj 8 DK BLU/WHT
78 
79 3-2 SS except 6.0L BRN or White
80 Sens rtn

RED connector

1 Ground BLK/WHT
2 TCC PWM BRN (to trans)
3 Fuel pump secondary relay control TAN (not used in my harness/conversion)
4 
5 Delivered Torque TAN/BLK  (to electronic brake control module) 
6 PC SOL HI RED/BLK (to trans)
7 EGR Control RED
8 PC Sol Lo LT BLU/WHT (to trans) 
9 Fuel Pump DK GRN/WHT (+12V power output for fuel pump relay, to fuse center)
10 RPM out WHT (tach output 4cyl signal to black 14 pin connector)
11 Recirc Sw DK BLU (to 2 pin connector on passenger side of harness) (A/C high press)
12 
13 Cruise Eng LT BLU/BLK (to black 14 pin connector)
14 
15 Gen L BRN (charge indicator control)
16 4WD Low GRY/BLK 
17 AC REQUEST DK GRN/WHT  
18 
19 
20 VSS Lo LT GRN/BLK (to VSS on side of trans)
21 VSS High PPL/WHT  (to VSS on side of trans)
22 Auto trans ISS high signal RED/BLK (not used on my harness/conversion)
23 Auto trans ISS low signal DK BLU/WHT (not used on my harness/conversion)
24 TP Sig DK BLU  (to TPS sensor)
25 IAT SIG TAN (to mass air flow temp sensor)
26 Ign Ctrl PPL/WHT (coil )
27 Ign Ctrl RED or RED/WHT (coil )
28 Ign Ctrl LT BLU/WHT (coil )
29 Ign Ctrl DK GRN/WHT  (coil ) 
30 
31 MAF SIG YEL
32 MAP SIG LT GRN
33 Recirc Door DK GRN 
34 Evap Can DK GRN/WHT (to solenoid on top of intake)
35 
36 
37 
38 
39 Ign Feed RED (cam pos sensor)
40 GND BLK/WHT
41 EGR GND WHT
42 TCC SOL TAN/BLK  except 6.0L (to trans)
43 A/C CLU DK GRN/WHT
44 
45 EVAP Vent WHT (pin C of blue 12 pin plug)
46 MIL Ctrl BRN/WHT (to black 14 pin connector chk engine light)
47 2-3 SS YEL/BLK
48 1-2 SS LT GRN 
49 VSS Out YEL/BLK (pin E of blue 12 pin connector)
50 VSS Out DK GRN/WHT (to black 14 pin connector)
51 TFT Sens Sig YEL/BLK (to trans fluid temp)
52 Gen F GRY (2000 only)
53 
54 Fuel Lev PPL (to pin K of blue 12 pin connector)
55 CYC SW in DK GRN (A/C system)
56 
57 SENS GND BLK 
58 
59 
60 REF Low BRN (to ign coils)
61 REF Low BRN/WHT  (to ign coils)
62 PRND C GRY (to trans)
63 TRANS RNG A PNK (to trans)
64 Fuel SIG CTRL DK GRN (pin D of 12 pin blue conector, to fuel tank pressure sensor)
65 
66 IGN CTRL PPL/WHT  (coil ) 
67 IGN CTRL RED/WHT  (coil ) 
68 IGN CTRL DK GRN  (coil ) 
69 IGN CTRL LT BLU   (coil ) 
70  
71 
72 
73 
74 
75 
76 IAC COIL HI LT GRN/WHT
77 IAC COIL LO LT GRN BLK
78 IAC COIL LO LT BLU/BLK
79 IAC COIL HI LT BLU/WHT

80 ORG/BLK (pin G of 12 pin blue connector)